A.	IDENTIFICATION

This batch record describes the production of Compound X from intermediate compound Y.  It is based on the process description provided in ___________.  Based on the amount of Y charged,  the approximate amount of Compound X expected can be calculated according to the equation below:

_______kg Y charged   x   1.35  =  _______ kg Compound X expected yield

# B.	PROCESS DESCRIPTION

Intermediate Y is reacted in toluene with AA and BB to form a mixed anhydride. CC is added, the mixture is heated and the formation of Compound X free base by esterification is monitored by HPLC. At completion, the reaction is quenched by washing with 10% aqueous monobasic sodium phosphate and water. The aqueous washes are discarded.

Following a polish filtration, the Compound X free-base solution is concentrated to approx. 16 wt%.  tBME  is added to adjust the concentration to approx. 8 wt%.  The hydrochloride salt is formed by the addition of 1.1 equivalents of  HCl by direct gas sparging.

The solution is heated to 50C and seeded with Compound X seeds.  The slurry is cooled, aged, and filtered to recover the the product.  The cake, a white to off-white crystalline solid, is washed with tBME and vacuum dried at 40-45C for approx. 12 hours, then at 55-65C until toluene <0.1%.

# C.	BILL OF MATERIALS

| Material | Mol Wt. | Part No. | Grade |
| --- | --- | --- | --- |
| Intermeditate Y | 234.3 | IM-0119 | 98% ee |
| Toluene | 92.14 | RM-0030 | 99.0% |
| AA | 99.18 | RM-0040 | 99% |
| BB | 136.58 | RM-0122 | 98% |
| Process Water | 18.0 | RM-0129 | DI |
| CC | 141.21 | RM-0037 | 97% |
| NaH2PO4•H20 | 137.99 | RM-0121 | 98% |
| Hydrogen Chloride Gas | 36.5 | RM-0026 | 99% |
| Compound X Seeds | 393.95 | RM-0056 | See spec. |

D.	RAW MATERIAL CHARGE TABLE

| Material | Step # | Factor | Amount Required |
| --- | --- | --- | --- |
| Intermediate Y | 2 |  | A = kg |
| Toluene | 4 | A x 5.2 | kg |
| AA | 6 | A x 0.84 | kg |
| BB | 8 | A x 0.59 | kg |
| CC | 10 | A x 0.79 | kg |
| 11.5% Aqueous NaH2PO4 | 15 | A x 6.4 | kg |
| Process water | 19 | A x 3.0 | kg |
| Compound X Seeds | 34 | A x 1.0 | g |
|  |  |  | Initials ________ |
|  |  |  | Checked _______ |

Note:   	Maximum operating volume is 13 liters per kg Intermediate Y charged.

Minimum operating volume is 1.5 liters per kg Intermediate Y charged.

E.	MAJOR EQUIPMENT

| - Ace 50-Liter Reactor - Buchi 50-Liter Reactor - Metering Pump - F-30 Hastelloy 18” Vacuum Filter - Busch vacuum system | - Filtrine chilling system - Vacuum Drying Oven - Mokon Heating System - 0.5µ Cartridge Polish Filter |
| --- | --- |

# F.	SAFETY DATA

1.	Review the Material Safety Data Sheets (MSDS) for each material listed above (see Bill of Materials) before starting any process work.

2.	Review the location of fire extinguishers, fire blankets, safety showers, spill cleanup equipment and protective gear before beginning any process work.

3.	During operations in the plant, the following safety gear will be utilized at all times:

- Lab Coat

- Safety Goggles

- Protective Gloves (See Table below)

| Material | ### Recommended gloves |
| --- | --- |
| toluene | nitrile, neoprene |
| Concentrated HCl | nitrile, neoprene |
| AA | neoprene or butyl rubber |
| BB | neoprene, butyl |
| MtBE | nitrile, PVA |
| HCl gas | nitrile, neoprene |

4.	Special note:  AA is a particularly hazardous material.  Avoid inhalation of vapors and wear neoprene or butyl rubber gloves.  Neutralize spills by sprinkling with sodium carbonate.

Operator Sign-Off:

I have read and understood the safety information provided above, including MSDS’s and reviewed the location of the recommended safety equipment:

Operator signature					Date

____________________________________	_____________

____________________________________	_____________

____________________________________	_____________

____________________________________	_____________

____________________________________	_____________

____________________________________	_____________

# G. 	PROCEDURE

| 1. Inert the 50-L Buchi reactor with Argon. |  | Date: __________ Time: __________ Init: __________ |
| --- | --- | --- |
| 2. Charge the reactor with ______ kg Intermediate Y. G__________ Intermediate Y lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 3. Re-inert the reactor with Argon. Note: Avoid allowing solids to be sucked from the reactor into the vacuum line. |  | Date: __________ Time: __________ Init: __________ |
| 4. Charge the reactor with ______ kg of toluene. G__________ Toluene lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 5. Begin agitation. Agitation %: ____________ |  | Date: ________ Time: ________ Init: ________ |
| 6. With mixing and cooling, at 20-30C, slowly add ______ kg AA to the reactor. AA should be charged using a peristaltic metering pump using viton tubing or via a clean, dry pressure pot. A mild exotherm is anticipated; record any changes in temperature. AA lot# or ID#: _____________________ Added Date/Time(hr) TemperatureC Agitation % Amount Notes ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ Init: _________ Check: _________ |  |  |
| 7. Rinse the addition line by following with toluene as necessary. G__________ Toluene lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 8. With mixing and cooling, at 20-30C, slowly add ______ kg BB to the reactor. BB should be charged using a peristaltic metering pump using viton tubing or via a clean, dry pressure pot. A mild exotherm is anticipated; record any changes in temperature. BB lot# or ID#: _____________________ Added Date/Time(hr) TemperatureC Agitation % Amount Notes ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ ___________ _________ _________ ______ ______________________________ Init: _________ Check: _________ |  |  |
| 9. Rinse the addition line by following with toluene as necessary. G__________ Toluene lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 10. With mixing, at 20-30C, slowly add ______ kg CC to the reactor. CC should be charged using a peristaltic metering pump using viton tubing or via a clean, dry pressure pot. No exotherm is anticipated. G__________ CC lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 11. Rinse the addition line by following with toluene as necessary. G__________ Toluene lot# or ID#: Amount Charged: T__________ _______________________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 12. Maintain the contents of the reactor at 20-30C and mix for a minimum of 30 min. Collect a sample of the reaction mixture for HPLC analysis. Date/Time(hr) TemperatureC Agitation % Initials Notes/Sample ID ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ Init: _________ Check: _________ |  |  |
| 13. Heat the contents of the reactor to 60-70°C (target 65C). Continue mixing at this temperature and sample periodically for HPLC until the mixed anhydride area is less than 1%. Date/Time(hr) TemperatureC Agitation % Initials Sample/Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ Note: The reaction is monitored by HPLC method # 01263. |  |  |
| 14. Cool the contents of the reactor to 20-30C. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ Note: Reaction may be safely held overnight under argon at this point. |  |  |
| 15. Charge the reactor with ______ kg 11.5% NaH2PO4H2O aqueous solution. 11.5% NaH2PO4H2O solution G__________ Batch Number:___________ Amount Charged: T__________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 16. Mix the contents of the reactor at 20-30C for a minimum of 10 min. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 17. Stop the agitation and allow the phases to separate for a minimum of 10 minutes while maintaining the temperature at 20-30C. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 18. Split the phases and discard the aqueous (bottom) phase after sampling. Record the organic phase volume and aqueous phase weight. Aqueous phase weight: _______ Sample #: ________ pH: ________ Organic phase volume: _______ |  | Date: ________ Time: ________ Init: ________ |
| 19. Charge the reactor with ______ kg process water. G__________ Amount Process Water Charged: T__________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 20. Mix the contents of the reactor at 20-30C for a minimum of 10 min. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 21. Stop the agitation and allow the phases to separate for a minimum of 10 minutes while maintaining the temperature at 20-30C. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 22. Split the phases and discard the aqueous (bottom) phase after sampling. Record the organic phase volume and aqueous phase weight. Aqueous phase weight: _______ Sample #: ________ pH: ________ Organic phase volume: _______ Note: Reaction may be safely held overnight under argon at this point. | Date: ________ Time: ________ Init: ________ |  |
| 23. Transfer the organic phase into the Ace 50L reactor. Note: Ensure that the Ace is clean and dry prior to using. | Date: ________ Time: ________ Init: ________ |  |
| 24. Rinse the Buchi 50L reactor with toluene. toluene ID: ___________ Amount Used: kg | Date: ________ Time: ________ Init: ________ |  |
| 25. Connect a suitable 50µ pre-filter in-line from the Ace to the Buchi 50L reactor. Polishing Filter Used: _________________________________ | Date: ________ Time: ________ Init: ________ |  |
| 26. Polish filter the batch to the Büchi reactor and follow with toluene as needed for a rinse. toluene ID: ___________ Amount Used: kg Final filtered volume: _________________ | Date: ________ Time: ________ Init: ________ |  |
| 27. Collect a sample of the organic phase from above for quantification of Compound X free base and determination of density: Sample #: _________ Concentration: __________ Density: _________ Note: Compound X is quantified by HPLC method # 02267. Typical concentration is 18% by weight. | Date: ________ Time: ________ Init: ________ |  |
| 28. Calculate the amount of Compound X free base present in the batch: Compund X free base concentration (step 27): _____________ Solution volume (step 26): _______ Density (step 27): _______ Calculate organic solution weight : ________________________ Calculate total Compund X free base: ___________________ | Date: ________ Time: ________ Init: ________ Check: _______ |  |
| 29. Distill, at atmospheric pressure, the organic phase to approximately half the volume recorded in step 26. Typical boiling point is 110°C. Target volume : _______ L. Date/Time(hr) Temp. C Jack. TC Press Volume Agitation % Initials/notes _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ _________ _______ ________ _____ _______ _______ ___________________ |  |  |
| 30. Collect a sample of the organic phase for quantification of water by Karl Fisher and determination of density. Sample #: ___________ KF Result: _____________ Volume: ___________ Density Result: _____________ Note: Water content must be no greater than 0.2 wt%. Otherwise it is necessary to add back toluene and continue distillation until water content meets spec. |  | Date: ________ Time: ________ Init: ________ |
| 31. Charge the reactor with sufficient toluene to achieve Compound X free base concentration of 10% weight/vol. Required amount of toluene: __________L x 0.865 = ____________ kg toluene ID: G__________ ________________________ Amount Charged: T__________ N__________ Final Volume: _______________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 32. Based on the amount of Compound X free base determined in step 27, with mixing and cooling as necessary, slowly (over ~30 min) charge the reactor with 1.0 equivalents of HCl gas via subsurface sparging while maintaining the temperature at 20-35C. At 1.0 equivalents, sample for pH (pH should be 2-3). Total Compound X free base (step 27): ___________ Calculate required amount of HCl: 1.0 equiv = kg HCl gas ID : ____________ Init: ________ Check: __________ Time HCl added Temp Agitation pH Notes/Init __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ __________ __________ _________ _________ _____ _____________________ |  |  |
| 33. Adjust the temperature of the reactor to 40 - 50C. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 34. Seed the reactor with ______ g Compound X seeds prepared in a slurry with toluene (5-10 wt%). Compound X seeds G__________ Lot Number:__________ Amount Charged: T__________ N__________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 35. Mix the contents of the reactor at 40 - 50C for a minimum of two hours. Adjust mixer speed as necessary to maintain vigorous agitation. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 36. Cool the contents of the reactor to 20-25C over a minimum of one hour. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ Cooling profile followed: _______________________________________________ |  |  |
| 37. Hold the contents of the reactor at 20-25C for a minimum of two hours. Date/Time(hr) TemperatureC Agitation % Initials Notes ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ ___________ _________ _________ ______ _________________________________ |  |  |
| 38. Filter the mixture using the 18” vacuum filter F-30. Notes: ___________________________________________________________ _________________________________________________________________ |  | Date: __________ Time: __________ Init: __________ |
| 39. Wash the cake with a minimum of ______ kg MtBE at ambient temperature. G _________ MtBE Lot Number:__________ Amount Used: T__________ N _________ |  | Date: __________ Time: __________ Init: __________ Check: __________ |
| 40. Sample the pooled mother liquors/wash for density measurement and quantification of Compound X. Record total ML weight or volume. Mother Liquor wt: ________ Sample # _________ Density: ___________ |  | Date: __________ Time: __________ Init: __________ |
| 41. Record the weight of the wet cake isolated above. G__________ Wet Cake Weight: T__________ N__________ Collect a sample of the washed wet cake for analysis by HPLC. Wet Cake Sample #: ________________ |  | Date: ________ Time: ________ Init: ________ |
| 42. Transfer the solids from the buchner funnel to drying trays. Dry the solids under vacuum (>28in Hg) at 45C 2.5C for a minimum of 12 hours. Date Time (hr) Temp. C Press. (in Hg) Gross Weight Notes/Initials _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ |  |  |
| 43. Dry the solids under vacuum (>28in Hg) at 60C 2.5C for a minimum of 6 hours. Date Time (hr) Temp. C Press. (in Hg) Gross Weight Notes/Initials _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ _______ _______ _______ ________ ____________ ___________________ |  |  |
| 44. Weigh the dried solids and record the weight. Package the material in an appropriate container and label. Collect a sample (~5 g) for analysis. G __________ Dry Weight: T __________ Sample ID #: __________ N __________ Sample Wt: __________ |  | Date: ________ Time: ________ Init: ________ |
| 45. Calculate the yield according to the following formula: %Yield = (Product wt / MW 236.8) / (Intermediate Y charge / MW 188.8) x 100 = ________% Init: _______ Check: __________ |  |  |

# H.	BATCH RECORD CHECKLIST

- All sections signed off

- Analytical Results Recorded

- Analytical Data, Chromatograms, etc (if applicable) attached.

- Yield Calculated

- Equipment Cleaned

Notes: ____________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

_____________________________		_______________________________

Plant Supervisor	/   Date				Checked by		/  Date